| Description | Quantity | UOM |
| --- | --- | --- |
| WWTP clarifier complete pump Flow 18 [m3/hr.] ,Head 35 [m] ,Suction flange 2[in] ,Discharge flange 1 1/4 [in] ,used for salt water 10000-15000 PPM TDS | 2 | Each |
| WWTP MMF back wash complete pump Flow 72 [m3/hr.] ,Head 20 [m] ,Suction flange 3 [in] ,Discharge flange 2 1/2 [in] ,used for salt water 10000-15000 PPM TDS | 2 | Each |
| WWTP UF complete pump Flow 16 [m3/hr.] ,Head 30 [m] ,Suction flange 2 [in] ,Discharge flange 1 1/4 [in] ,used for salt water 10000-15000 PPM TDS | 2 | Each |
| WWTP UF back wash complete pump Flow 50 [m3/hr.] ,Head 20 [m] ,Suction flange 3 [in] ,Discharge flange 2 1/2 [in] ,used for salt water 10000-15000 PPM TDS | 2 | Each |
| WWTP RO flush complete pump Flow 20 [m3/hr.] ,Head 30 [m] ,Suction flange 2 1/2 [in] ,Discharge flange 2 [in] ,used for RO water PH 5-6 | 1 | Each |
| WWTP transfer complete pump Flow 20 [m3/hr.] ,Head 60 [m] ,Suction flange 2 [in] ,Discharge flange 1 1/4 [in] ,used for RO water PH 5-6 | 2 | Each |
| WWTP CIP complete pump Flow 20 [m3/hr.] ,Head 30 [m] ,Suction flange 2 1/2 [in] ,Discharge flange 2 [in] ,used for water with acid and alkaline PH 1-12 | 1 | Each |
| WTP2 Neutralization pit self priming pump with motor Flow 40 [m3/hr.] ,Head 50 [m] , Water with up to{ 5% Sulfuric Acid (H2SO4) ,5% Caustic Soda (NaOH) } | 2 | Each |